Performance Evaluation

The form below shows the execution time for running mygrep and UNIX grep for specified keywords. Compare to UNIX grep (which is a sequential program on CPU), we couldn’t see any speedup due to several reasons. First is the low I/O transfer speed. A lot of overhead is cost on copying data from CPU memory to GPU memory and copying back. Second is the GPU has very limited memory space for application defined allocation. And the speed is affected by the number of threads running at each time. From the form we can figure out that a searching process by 1536 threads is much faster than 256 threads.

| program | Time used (seconds) |  |
| --- | --- | --- |
|  | Keyword = 1925 | Keyword = word |
| Mygrep(6*256 threads) | 26.03 | 88.98 |
| UNIX grep | 4.90 | 43.84 |
| Mygrep (1*256 threads) | 208.16 | 216.37 |